Step 1. Connect USB-A side of the USB-A to USB-C line to PC first.

Step2. Press and hold the “SWITCH” key, then connect USB-C side of the USB-A to USB-C line to unit, Press and hold the key for three seconds, then release it.

You will see the “HDMI OUT” led is flashing like below.

Step3. Run the tool “GD32AllInOneProgrammer.exe” on PC, then Select “USB”, Click “Connect”, like below.

Step4. Select “Necessary erase”. And load the firmware “EZ-SW21HA21_Ver1.13_APP(46D7).hex”

(Note: It is very important to select “Necessary erase”.)

Step5. Click “Download” to upgrade the firmware, After the upgrade is complete, you will see that ”Finish”

Step6. Re-power the unit, and it can start to work.